Voxx, The 16-Band Vocoder for ATARI Falcon 030

(c)1994 Hans Achim Mueller

Short User's Manual for Demo_Version.

INDEX

-----

1.- Introduction

2.- Installation & DEMO-Restrictions

3.- Description

4.- Operation

5.- Some Useful Hints

6.- Specifications

7.- Availability

1.-     Introduction

This is the Demo-Version of a 16-Band Vocoder effect for the ATARI

Falcon030 computer. This program is Public Domain and free of charges.

It can be copied and distributed, if it is done entirely, without any

changes in the code or this text! Any distribution of mutations,

improvements, cracks etc. of this program without my written permission

is illegal!

*************************** IMPORTANT *********************************

I do not assume any responsability, in any form, for direct or indirect

damages, that may result from the use of this programm!

***********************************************************************

The german version of this text will follow soon (February).

2.-     Installation

This Demoversion of Voxx should have the following files:

File Name      Lenght

VOXXDEMO.PRG   46653

VOXXMENU.RSC    2454

VOXX_UPR.RSC   52006

VOXXENV0.RSC   65438

VOXXENV8.RSC   61982

VOXX_PAN.RSC   49128

VOXXMIX0.RSC   65438

VOXXMIX8.RSC   61982

VOXX_DOC.TXT   (This text)

Be sure these files are the same length (where specified) in other case

you have a modified and therefore an ILLEGAL copy!

Voxx requires 512 KB of free memory and runs only on ATARI Falcon030 wih

a RESOLUTION of at least 640 * 480 and at least 16 COLORS! (VGA Monitor

or similar). If you do not have these requirements please write me a

postcard telling your configuration in order to check if it will be

available and when.

IMPORTANT: Before you start the program please read carefully this

documentation to avoid startup problems (there are some) that I am trying

to solve now. Furthermore a Vocoder is a complex effect that requires

some hints and knowledge to satisfy that what is expected from it!

2.1.-   Restrictions in the DEMO-Version:

The purpose of a Demo-version is to let the user take an idea of what a

program can be or do. After deep thoughts about what should be in the

Demo-version of Voxx, I decided to release it at the conclusion of it's

developement. I could have released a demo much earlier, but it's screen

would not have been so impressive like it is now!. Actually the

Beta-version is ready and this demo is a slightly restricted version of

it. The restrictions are:

a.- The options marked with *** are disabled and these are: the Load and

Save File options, Program Store.

b.- The program changes (cycles) every 30 seconds to the next user

program. The user programs are uneditable presets (program Store

disabled) and some  programs are intentionally missadjusted to

sound not so good. User adjustments on the actual settings are

possible at any time, but at least in 30 seconds will be overwritten

by an automatic selected program. If any accesory is selected and

the 30 seconds count is reached the accesory window will be

overdrawn by the new adjustements. This is intentionally and

harmless. Closing the accesory you get a clean Voxx-screen again.

To run, copy all files to a FOLDER and be sure the PATH is set to the

application when starting the program.

3.-     Description

For all those who do not know what is a Vocoder and how it operates, or

those that not have a lot of experience with them, I will try to explain

it as simple as I can: Apparently the Vocoder is a Military residual

product that was developed to encode the speech in a way that it was

impossible to be understood by other listeners until they also had a

Vocoder with the same configuration. The function of a Vocoder is to

split the speech (FORMANT) or any sound into several Frequency-Bands

(eg. to analyze each band) and apply the amount of signal in each band

to another sound (CARRIER), that also is splitted in Frecuency-Bands. In

other words, the spectrum of the speech was used to FORM the sound of

the CARRIER. The most analog Vocoder is our own Vocal-System that is

composed of:

a.-Vocal chords: who produce the voice (Oscillators)

b.-Mouth: that forms the vocalization (Filtering and Noise)

The vocal chords produce a sound (voice) that only can vary the pitch

but not the sound character. The waveform of this sound is nearby a

Pulse-Wave eg. rich in Harmonics. The sound character is given by our

mouth formation (Vocalization). The form and volume of our mouth-cavity

(including tongue position and mouth aperture) produce a resonance

cavity that acts like a filter on the voice.

Any variation of our mouth is a variation in the filter parameters, so

we can say that we have a Time-Variable-Filter of more complex structure

than known of analog synthesizers. In addition the same airflow that

excites the vocal chords also can be/is used to create several noises

("s", "r", "t", etc) when it passes the throath, tongue and teeths,

which are added to the voice and also filtered.

These variations, slower than the voice's pitch, are variations in the

frequency-spectrum of our voice. If these spectrum-variations are

extracted and used to parametrize the spectrum of a sound, similar to

our voice, we get a synthesis of our voice. This synthesis will be as

good as the spectrum-analisys and reapplication to the sound are.

A well known example (at least for musicians) is the Jew's harp where

the instrument creates the sound (voice) and the mouth does the

vocalization.

A Vocoder is composed of following main sections: The analisys section

(FORMANT) that extracts the spectrum variations of the vocalization

source (speech, drums, etc), the voice section (CARRIER) where these

variations are applied to the target sound (strings, pulsewaves, noises

etc), something to interconnect both sections (routing MATRIX) and other

sections, for signal conditioning like GAIN, MIX, PAN and VU-Displays

etc. The names in uppercase are those I used in Voxx.

3.1.-   Formant section:

The formant section is composed of 16 Band-Pass-Filters (BPF) for

different frequencys (like an Equalizer) covering the desired frequency

spectrum and Envelope Generators (EG's). Each BPF lets pass-through the

part of the input signal's spectrum that lies around the BPF's center-

frequency (Fc) attenuating the rest of the signal's spectrum

proportionally to it's "distance" to The BPF's Fc. In other words this

stage "slices" the Formant signal in frequency bands. The more bands

(BPF's) a vocoder has the more precise is the spectrum characteristic it

can analyze. Each band output ("slice") passes in the next stage to an

envelope generator to form a envelope for each band, proportional to the

formant signal's spectrum. As mentioned before, the vocalization is slow

in relation to the sound, threfore envelopes are used instead of the

direct BPF's output. The Attack/Decay time of The EG's is adjustable via

the ENV-Faders to create different effects of vocalization.

3.2.-   Carrier section:

This section also has 16 BPF with the same Fc's as the formant section

and again here the the carrier signal is filtered and "sliced" into

spectrum components but this time the BPF's outputs go to

Value-Controlled-Amplifiers (identical to synthesizer VCA's but

controlled by digital Values, not Volts). The control value for these

amplifiers is supplied by the Formant Envelopes. The result is that each

Carrier-band output is modulated in amplitude by the amount of signal of

this band in the Formant signal. If all results are added (MIXed)

together the result is a "symbiosis" of the "speech" of the formant with

the "voice" of the carrier.

This is the principle that makes Talking Instruments talk! (or sing?)

3.3.-   MATRIX:

In the beginning I mentioned the military purpose of a Vocoder to code

spoken messages and it was based on arrange the routing (Patching) of

formant envelopes to the carrier in an unnatural order to make the

speech/voice not understandable. Only with the reverse routing it would

be possible to reconstruct partially the original speech. I do not know

if this was usefull anytime for these purpose, but for musicians and the

movie-FX-sections (Greetings from Robot-Worlds) I am sure it is . This

routing is done in Voxx via the MATRIX an it is intended to route the

formant envelopes to the carrier amplifiers.

3.4.-     Additional sections (at least for Voxx):

I/O-Section:

This section is for input and output signal gain adjust and level

display.

SPECTRUM & settings section:

This section displays the formant spectrum envelopes and allows some

global settings for envelope modes.

MIX/PAN section:

The resulting signal is adjustable in this section via the MIX faders

and the PAN knobs for each band similar to an equalizer.

4.-     OPERATION:

This long description (I hope, I have reached something more than

confuse you) is followed now by the operation of Voxx.

4.1.-   Connecting the Falcon030 :

IMPORTANT!

The Falcon030 audio inputs are designed for microphone signals and very

sensitive. If you connect them with a LINE-Output of instruments,

CD-players or other audio devices be carefull with the levels. In this

case the better way is to insert 200K Resistors in series with the

audio-source and the Falcon030 Inputs as in Fig. 1 to reduce Noise and

Humm. For more information about this, contact ATARI Corp. or your local

dealer.

Fig. 1

.......................................................................

:                                                                     :

: Instrument                         Resistor       Falcon030 Inputs  :

:                                                                     :

:                  Shielded Cable   +------+                          :

:  Line-Out-Left  ==================| 200K |=============-  Left      :

:                                   +------+                          :

:                                                                     :

:                  Shielded Cable   +------+                          :

:  Line-Out-Right ==================| 200K |=============-  Right     :

:                                   +------+                          :

:                                                                     :

:                                                                     :

:.....................................................................:

To connect the Falcon030 for use as Vocoder effect you need:

FORMANT sound source like a MICROPHONE, Tape deck, musical instrument,

etc, connected to the Left-Input of the Falcon030.

CARRIER sound source  like SYNTHESIZER, elctric guitar, etc connected to

the Right-Input of the Falcon030.

A headphone, amplifier etc, connected to the Falcon030 stereo output.

NOTE: Uppercase devices are the most used in a Vocoder effect (see

chapter 5.)

4.2.-   Voxx-Screen:

The following is a blockdiagramm of the Voxx Vocoder as it is arranged

in the screen:

------------------------------------------------------------------------

"Desk"  "File"                   Menu Bar

------------------------------------------------------------------------

I/O Section          Spectrum & settings                 Matrix

+------------+  +----------------------------+  +----------------------+

| ####  #### |  | ########################## |  |  F                   |

| ####  #### |  | ########################## |  |  E                   |

| ####  #### |  | ########################## |  |  D                   |

| ####  #### |  | ########################## |  |  :                   |

| ####  #### |  | ########################## |  |  :                   |

| F  C  L  R |  |                            |  |  2                   |

| O  A  O  O |  |  P1 P2 P3 P4 P5 P6 P7 P8   |  |  1                   |

| R  R  U  U |  |                            |  |  0                   |

| M  R  T  T |  | FREEZE SLOW FAST STORE_PRG |  | RST 0 1 2 .... D E F |

+------------+  +----------------------------+  +----------------------+

+-------------------------------------------------------------+ +------+

|  E                                                          | | Voxx |

|  N                     16 ENV-Faders                        | |  F   |

|  V                                                          | |  U   |

+-------------------------------------------------------------+ |  T   |

+-------------------------------------------------------------+ |  U   |

|  PAN                   16 PAN-Knobs                         | |  R   |

+-------------------------------------------------------------+ |  E   |

+-------------------------------------------------------------+ |      |

|  M                                                          | |  U   |

|  I                     16 MIX-Faders                        | |  S   |

|  X                                                          | |  E   |

+-------------------------------------------------------------+ +------+

"#" = Displays

4.3.-   Menu bar

The menu bar is composed of the titles "Desk" and "File"

4.3.1.- "Desk" is the menu for the options:

-  About Voxx..: which shows the program's version and Users

Registration.

-  The other options are the accesories your system has installed.

4.3.2.- "File" is the menu for Loading, Saving a file and Quit the program.

***     -  Load File: Loads a file with the settings for 8 user programs

and the previous saved memory settings.

***     -  Save File: Saves a user file with the 8 user programs as well

as the actual memory settings.

NOTE: The file extensions are predefined as "VXD" (VoxX Data)

-  Quit: Exits the program.

4.4.-   I/O Section

The I/O section is  composed of 2 VU-Displays for Input and Output

levels, Left and Right each. The left input of the ATARI Falcon030 is

for the FORMANT (speech) and the right input for the CARRIER (voice

instrument). The output is the stereo effect (Left and Right). The 4

faders below are for adjustment of the input and output levels. To move

a fader, point it with the mouse and press the left button, holding it

(the arrow will change to a hand), now you can move the mouse up and

down to adjust the selected level. The slider will follow within his

limits while the left mouse button is pressed.

4.5.-   Spectrum and settings section

The large display, shows the actual envelope value for each CARRIER-band

that is extracted from the Formant source, like a spectrum analyzer. The

frequncy for each band is displayed below. The program buttons allow the

selection of 8 user setting programs that are loaded at the start of the

program or via the "Load File" option in the Menu title "File". These

are user programs. Each of these programs holds the settings for all

adjustable parameters. Pressing a program button, the selected program

is copied to the edit memory . Each change in the adjustements is done

in the edit memory (those that are active). If you want to store these

settings for future use you must write them to one of the 8 user

programs (see STORE PRG) and then save them with he option "Save File"

in the menu title "File".

Of the 4 buttons at the bottom the 3 named FREEZE, ENV SLOW and ENV FAST

are for a global range setting of the formant envelope attack/deacy

velocity.

FREEZE sets all attack/decay velocities to zero regardless of the ENV

fader settings, freezing all actual envelope values until another

button, SLOW or FAST is pressed. This function is usefull to equalize

the CARRIER with a FORMANT's spectrum characteristic.

SLOW and FAST set two different envelope velocity ranges with a relation

1 to 10.

***     The STORE PRG button is to store the actual settings (in memory) in one

of the 8 available programs. To store a program press STORE PRG, all

the program buttons will then show pressed and then press the program

button where you wish to save the settings. The settings will be

recalled each time you press the selected program button.

4.6.-   The MATRIX

The matrix is used to connect the FORMANT Envelopes to the CARRIER

amplifiers. For each band there is a FORMANT(source) button(green) and a

CARRIER(target) button (yellow) labeled from 0 to F, where the 0 stands

for the lowest frequency band and F for the highest. With this matrix it

is possible to assign or route the envelope of one determinated

frequency band to any of the CARRIER-band-amplifier. So you can assign a

low frequency amount in the FORMANT to modulate a high frequency band of

the  CARRIER and vice versa. In a 1 to 1 routing the effect of a talking

instrument is the most notable while the shifting of the bands in both

directions by one or two (0 with 1, 1 with 2, ... E with F  or

vice versa) adds the effect of pitch shifting (which is the logical

consequence of shifting the bands). Very scarammbled routings may result

in interesant and unespected effects of vocalization that surely will

not be understood as human speech, regardless of what source for the

FORMANT is used.

The RST button ReSTores the 1 to 1 routing of the MATRIX for comfort.

To operate in the matrix is very simple, then once the mouse enters the

routing field, it's pointer transforms in a cross that intersects with

both the FORMANT and CARRIER band buttons. If you want to connect

FORMANT band 5 to CARRIER band E place the cross in the position where

the green button "5" and the yellow button "E" are pointed by the

horizontal and the vertical lines respectively, then press the left

mouse button and the routing will be done. You can do this routing as

you like. A FORMANT source can be routed to one or more CARRIER targets

but a CARRIER target can only recive one source. This means that if a

source is routed to more than one target there will be one or more

sources without a respective target.  Leaving the routing field returns

the mouse pointer to the usual arrow.

4.7.-   ENV section

This section consists of 16 faders each one assigned to a frequency

band. According to the range selected with the ENV SLOW/FAST buttons it

is possible to adjust the attack/decay for each FORMANT envelope. The

lowest slider position is for slowest and the highest for fastest

attack/decay times. These sliders are moved by pressing the left mouse

button while it is positioned in one of the faders and keeping the

button pressed moving the mouse up and down (pointer is now a hand).

A speciality for this section as well as for the MIX section is that if

you move the mouse pointer over another fader of this section while the

button is pressed, then the new pointed fader becomes active leaving the

one before in the last position. With this method it is possible to

"draw" the slider positions with the mouse, moving it over the entire

fader section. In this way it is possible to slide all faders down or up

by entering in one fader, pressing the left mouse button and holding it,

to travel the mouse pointer accross the entire screen. While the mouse

button is pressed the only affected faders will be those of the section

where the mouse pointer was in the moment the left button was pressed.

4.8.-   PAN section

Since the FORMANT (left input) and the CARRIER (right input) are used

both to form one signal, this section is for adjusting the stereo

panorama position for each modulated CARRIER band (result output), to

create a stereo effect.

It is provided with 16 knobs (potentiometer simulations), one for each

band. To move one of these knobs move the mouse pointer to it and press

the left mouse button and while holding it, move the pointer around the

knob. While the left button is pressed the knob mark will point in the

direction of the mouse pionter over the entire screen.

4.9.-   MIX section

The result of each coded band now is ready and with this section it is

possible to adjust the gain for each band (equalize) to fit the desired

tone characteristic. The MIX faders are adjusted in the same way as the

ENV faders.

Additional information: When any adjusting element is being changed

(faders and knobs), each variation is inmediatly send to the DSP (and

not when the left mouse button is released). You will hear the changes

in realtime (as fast as realtime is possible). This has a disadvantage:

The faders are a little slower than those that are commonly used to set

parameters (here the information is actualized after the change/movement

is completed by releasing the mouse button. I think that for a musical

effect, realtime is a more important factor than fast slider animation.

Recalling a program also involves sending all parameters to The DSP.

5.-     Some Useful Hints

Experience has shown that the use of a vocoder could be frustrating for

some users due to bad understanding of it's operation and what the user

wants to do with it. My shortest explanation of a vocoder I ever had

given to those who asked me "what is a vocoder?", was:

"it is like a good equalizer acting over a sound (CARRIER) whose

gain controls are fastly moved by a device that extracts the

movement information from the frequency spectrum of another

sound or speech (FORMANT)."

To obtain a good vocoder effect it is important to know what kind of

effect is expected and what FORMANT and CARRIER sound sources are used.

The most applications use speech as a source for the FORMANT and

synthesizer-sounds for the CARRIER.

The FORMANT sources like speech should be clear and cover the most part

of the spectrum (use the display). A thin voice like through a phone or

a smooth one (like mine) are very poor in the spectrum's "Skyline". A

very good speech source is that of the News Speaker in the FM/UKW-Radio

(They have all the equipment to fill the spectrum with speech). I

hearthly recommend this test! If using your own voice try not to sing or

vary the pitch according to the music (else your spectrum moves with or

against the spectrum of the carrier creating unregular highs and lows in

level). Remember that the most part of the voice (sound) of the FORMANT

is discarded, depending on the envelope settings, and only the

vocalization is extracted. Drums or other percussions are also used as

formant to create rythmic vocalizations of an instrument. Here the

limits and settings are unlimited and depend of your imagination.

CARRIER sources should be rich in harmonics and these are all those

clear and sharp sounds like Strings, Brass and all those sounds whose

waveform is based on sawtooth and pulses. Square, triangle and sine

waveform based sounds (smooth ones) like Flutes, Whistles and some

organs aren't good ones. Their harmonics are very small in relation

with the fundamental which will dominate the spectrum (a true sine has

no harmonics) and the resulting sound will be only in the bands near the

fundamental's frequency band. The CARRIER's pitch is the pitch for the

resulting effect, here is where the melody, chords or whatever should be

played.

I will try to list the effects and the best settings as well as sources

for each one:

a)  Talking (singing) instruments, Choirs, Backgrounds, Harmonization:

FORMANT : Human Speech

CARRIER : Strings, Brass, Sawtoths, Pulses playing the melody or

accompainement chords.

Settings: Envelope SLOW

ENV Faders: upper positions

MATRIX: 1 to 1 or shifted by one or two

or

Envelope FAST

ENV Faders: lowest positions (in upper positions FORMANT's

pitch adds to the result)

b)  Robots:

FORMANT : Human Speech

CARRIER : Strings, Brass, Pulses, Sawtoths,  with constant pitch

(no melody) or steady chords.

Settings: Like a)

c)  Solist Singer

FORMANT : Human Speech

CARRIER : Strings, Brass, Pulses, Sawtoths,  with monophonic

(one Finger) melody (synthesizer options like DUAL,

UNISON, FAT, PORTAMENTO etc have very good results)

Settings: Like a)

d)  Rythmic Chords and sounds

FORMANT : Drums, percussions

CARRIER : Strings, Brass, Pulses, Sawtoths, etc.

Settings: All Envelope settings (except FREEZE)

e)  Warm instrument contouring (variable filtering)

FORMANT : No limits

CARRIER : No limits

Settings: All Envelope settings (except FREEZE)

e)  Equalizer (fixed filtering):

FORMANT : Blows in the micrphone Noise or any sound

CARRIER : No limits

Settings: First envelope SLOW or FAST.

When the spectrum display reaches a desired

characteristic press FREEZE

or

using Noise or blowing the micro and pressing

FREEZE when spectrum display is equally leveled.

Use the MIX faders to equalize the sound.

6.-     SPECIFICATIONS

The following are the main Voxx specifications:

Number of Bands   : 16 FORMANT Filters and 16 CARRIER Filters

Filter Types      : Digital-IIR, 4-Pole Band-Pass

Center Frequencys : 125, 160, 210, 275, 360, 460, 600, 780, 1020,

1325, 1725, 2250, 2900, 3800, 5000 and 6400 Hz.

Sampling Rate     : 20.77 kHz, 16 Bit Stereo

DSP Usage         : 100%

Dimensions(Pixels): 630 x 460 x 1 Pixels (W x H x D)

Memory Consumption: aprox. 512 kB

7.-     AVAILABILITY of Voxx Version 1.00

Voxx will be available in it's version 1.00 in March 1994!

Voxx will consist of a Diskette with the program, a	printed

user's manual similar to this one, in German or English.

Support and Updating: I will try to support any problems that may appear

in the use of Voxx only to registered users, and to Update this

program. The Update prices for registered users will only be those

for material + handling.

In the Future: Depending on Voxx's acceptance I am planning to MIDIfy

Voxx, and for experienced users, add a Filter Editor to change the, BPF

parameters like Resonance and Center Frequency. Any written sugestions

are welcome and if possible will be considered!

Coming Next: Pterodaktilus The Synthesizer(s) for Falcon030

Problems and errors with the Demo-Version:

As far as I have tested and know, this demo runs without any notable

error. There may be some problem when starting Voxx: The Spectrum

Display will overdrive, at the output may be only noise and any attempt

to reduce the Levels will be without effect. Solution: Start again

and/or Cold Reset (Set amplifier inputs and volume low!). I am trying to

solve this problem. If any error or strange effect occur please let me

know! I will try to fix it. I have programmed and use Voxx myself with

my Falcon030, as it comes from factory, without changes, additions or

hardware upgrades with TOS 4.02 and 4.04.

Multitos and Voxx: I recommend to use Voxx in the normal TOS mode

(Single Task). Due to the intense communication  between Voxx and the

DSP, I doubt if there is time enough left for "Multitosing".

Prices will be:

Germany: DM 195,- (+ Porto und Verpackung, z.Zt. nur V-Scheck)

Rest of Europe: DM 195,- (+ Postage & Packaging, Eurocheque Only)

Contact to:

Hans Achim Mueller

Nadlerstr. 6

D-69214 Eppelheim

GERMANY

Distributor inquiries are welcome!

January, 1994